Line-drainage-system consisting of:

- Channel body made of FILCOTEN® HPC (High Performance Concrete),   standard length 1000 mm, with safety seam, with preformed bottom outlet
optional: 500 mm
Load Class A15kN – B125kN; tested according to EN1433; CE-certified
no crossbars in the channel cross-section
tested and recommended by the IBR (Institut für Baubiologie Rosenheim)

- Channel covers A15kN – B125kN depending on the requirements out of galvanized steel, stainless steel or ductile iron (GGG): slotted grating SW5/80, ductile iron mesh grating MW15/25, slot top

- Accessories:
sump unit: out of FILCOTEN® HPC (High Performance Concrete)
optional: silt bucket (plastic), odour trap for sump unit or bottom outlet (plastic)
front and end plates out of galvanized steel; with or without outlet

specification text:

- Prefabricated drainage channels, of FILCOTEN® HPC (High Performance Concrete) to load class B125kN, according to EN1433 - CE certified, IBR tested and not flammable (class A1)

with preformed bottom outlet and integrated anchorage back taper

NW100 Dimensions:

Construction length: 1000 mm (500 mm) width: 122 mm height: 95 mm  max. weight: 8,3 kg

- gratings made of galvanized steel or ductile iron with fixing clips, load class A15kN – B125kN,

according to EN1433 - CE certified

cl. A: slotted grating:		 length 1000/500 mm     SW5/80		galvanized steel

cl. B: ductile iron mesh grating:	 length 500 mm	      MW 15/25	ductile iron

Slot top asy mmetric made of galvanized steel or stainless steel, SW 12,5 mm, SH 80 mm, Load classes B125kN
cl. B: slot top: 			 length 1000 mm	     height 103 mm
Maintenance unit to slot top: 	 length 500 mm

Construction time barrier out of wood for SW12,5/18:     length 1000 mm

- sump unit out of FILCOTEN® HPC (High Performance Concrete)

Dimensions: length 500 mm, (WxH) 138x380 mm
outlet: DN 100 by pipe-coupling
optional: silt bucket and odour trap made of plastic

- front / end plates with or without outlet DN100, galvanized steel

- odour trap

for bottom outlet DN100 made of plastic

for sump unit DN100 made of plastic