- Drainage-channel BG-FILCOTEN tec NW 200 made of FILCOTEN HPC (high-performance concrete)

with integrated edge out of galvanised steel or stainless steel 1.4301,

load-class A15 – C250 kN, according to EN1433 – CE-certified,

channel joint with safety seam, sealable,

length 1000 mm, without slope

Building biology tested and recommended by IBR (Institut für Baubiologie Rosenheim, DE)

Fire-behaviour class: A1 according to EN13501 - non-combustible

100% recyclable with U-A marking

Delivery and installation according to the manufacturer's installation instructions.

NW/No.:		LxWxH: [mm]		weight: [kg]	eco-indicator: [kg]

Galvanised steel edge:

200 H195:		1000x230x195		30,2		CO2-Eq. 10,4

200 No. 0:		1000x230x245		34,0		CO2-Eq. 11,5

Stainless steel edge:

200 H195:		1000x230x195		30,2		CO2-Eq. 9,3

200 No. 0:		1000x230x245		34,0		CO2-Eq. 10,4

- Drainage-channel as described in position 1, but

overall length 500 mm, without slope

NW/No.:		LxWxH: [mm]		weight: [kg]	eco-indicator: [kg]

Galvanised steel edge:

200 H195:		500x230x195		15,1		CO2-Eq. ca. 5,2

200 No. 0:		500x230x245		17,0		CO2-Eq. ca. 5,8

Stainless steel edge:

200 H195:		500x230x195		15,1		CO2-Eq. ca. 4,7

200 No. 0:		500x230x245		17,0		CO2-Eq. ca. 5,2

- Surcharge for bottom outlet DN 200

- Covers/Grates made of galvanised steel, stainless steel 1.4301, cast iron or plastic with integrated longitudinal movement protection and fiX-lock, load class A 15kN – C250kN (D400-E600kN – design grates), according to EN1433 - CE-certified

BG-FILCOTEN tec/pro mesh grating NW 200, galvanised steel

cl. B125, MW 30/10, length 1000 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro mesh grating NW 200, galvanised steel

cl. B125, MW 30/10, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro mesh grating NW 200, galvanised steel or stainless steel

cl. C250, MW 30/10, length 1000 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro mesh grating NW 200, galvanised steel or stainless steel

cl. C250, MW 30/10, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro perforated grating NW 200, galvanised steel or stainless steel

cl. C250, DM 6, length 1000 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro perforated grating NW 200, galvanised steel or stainless steel

cl. C250, DM 6, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro ductile-iron bar grating NW 200, ductile-iron EN-GJS

cl. C250, MW 29/13, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron design-grate „VIA“ NW 200, ductile-iron EN-GJS

cl. D400, SW 7, length 500 mm

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron design-grate „KIARO“ NW 200, ductile-iron EN-GJS

cl. D400, SW 7, length 500 mm

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron design-grate „VILLE“ NW 200, ductile-iron EN-GJS

cl. E600, SW 7, length 500 mm

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

Anti-vandalism locking device made of stainless steel

cross piece for blocking the fiX-lock

- sump unit kit BG-FILCOTEN tec NW 100 made of FILCOTEN HPC (high-performance concrete)

with integrated edge made of galvanised steel or stainless steel 1.4301, incl. silt bucket

outlet: pipe coupling DN 200

length 500 mm / width 261 mm / height 650 mm

Delivery and installation according to the manufacturer's installation instructions.

Galvanised steel edge:	48,0 kg

Stainless steel edge:	48,0 kg

- front cap closed, made of galvanised steel or stainless steel 1.4301

weight: 0,8 kg

- end cap, with outlet DN100 made of galvanised steel or stainless steel 1.4301

weight: 0,5 kg

- installation chair, Kit incl. screws for positioning the channel, galvanised steel

weight: 1,7 kg