- BG-FILCOTEN step shoe-scraper box made of FILCOTEN HPC (high-performance concrete)

with integrated edge made of galvanised steel,

walkable and wheelchair accessible

Sizes: 600x400, 750x500 and 1000x500 mm

Building biology tested and recommended by IBR (Institut für Baubiologie Rosenheim, DE)

Fire-behaviour class: A1 according to EN13501 - non-combustible

100% recyclable with U-A marking,

Delivery and installation according to the manufacturer's installation instructions.

Size: LxWxH: [mm]		weight: [kg]

600x400x80			13,2

750x500x80			18,0

1000x500x80			25,0

- BG-FILCOTEN step mesh-grate-cover for shoe-scraper box, made out of galvanised steel,

MW 30/10mm

walkable and wheelchair accessible

Size: LxWxH: [mm]		weight: [kg]

600x400x20			4,8

750x500x20			6,7

1000x500x20			9,4

- BG-FILCOTEN step shoe-scraper mats with load-bearing aluminium profile,

type rib-profile, colour anthracite

walkable and wheelchair accessible

Size: LxWxH: [mm]		weight: [kg]

600x400x20			3,3

750x500x20			5,3

1000x500x20			7,0

- BG-FILCOTEN step shoe-scraper mats with load-bearing aluminium profile,

type rubber-profile, colour black

walkable and wheelchair accessible

Size: LxWxH: [mm]		weight: [kg]

600x400x20			4,6

750x500x20			7,4

1000x500x20			9,8

- odour trap for bottom outlet DN 100, plastic

weight: 0,2 kg