- Drainage-channel BG-FILCOTEN pro mini NW 200 made of FILCOTEN HPC (high-performance concrete)

with integrated edge made of galvanised steel, stainless steel 1.4301 or cast iron,

load-class A15 – E600 kN, according to EN1433 – CE-certified,

channel joint with safety seam, sealable,

length 1000 mm, without slope

Building biology tested and recommended by IBR (Institut für Baubiologie Rosenheim, DE)

Fire-behaviour class: A1 according to EN13501 - non-combustible

100% recyclable with U-A marking

Delivery and installation according to the manufacturer's installation instructions.

NW/No.:		LxWxH: [mm]		weight: [kg]	eco-indicator: [kg]

Galvanised steel edge:

200 BH 100:		1000x238x100		21,3		CO2-Eq. 9,7

200 BH 120:		1000x238x120		23,2		CO2-Eq. 10,2

Cast iron edge:

200 BH 100:		1000x238x100		22,8		CO2-Eq. 12,3

200 BH 120:		1000x238x120		24,4		CO2-Eq. 12,9

Stainless steel edge:

200 BH 100:		1000x238x100		21,3		CO2-Eq. 7,6

200 BH 120:		1000x238x120		23,2		CO2-Eq. 8,2

- Surcharge for bottom outlet DN 200,

for drainage channels without slope

- Covers/Grates made of galvanised steel, stainless steel 1.4301 or cast iron with integrated longitudinal movement protection and fiX-lock or 4-fold screw connection, load class A 15kN – E 600kN, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro mesh grating NW 200, galvanised steel or stainless steel

cl. C250, MW 30/10, length 1000 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro mesh grating NW 200, galvanised steel or stainless steel

cl. C250, MW 30/10, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro mesh grating NW 200, galvanised steel or stainless steel

cl. D400, MW 25/10, length 1000 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro mesh grating NW 200, galvanised steel or stainless steel

cl. D400, MW 25/10, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro perforated grating NW 200, galvanised steel or stainless steel

cl. C250, DM 6, length 1000 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro perforated grating NW 200, galvanised steel or stainless steel

cl. C250, DM 6, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN tec/pro ductile-iron bar grating NW 200, ductile-iron EN-GJS

cl. C250, MW 29/13, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron bar grating NW 200, ductile-iron EN-GJS

cl. D400, MW 29/13, length 500 mm,

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron slotted grating NW 200, ductile-iron EN-GJS

cl. E600, SW 14/200, length 500 mm

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron design-grate “VIA“ NW 200, ductile-iron EN-GJS

cl. D400, SW 7, length 500 mm

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron design-grate “KIARO“ NW 200, ductile-iron EN-GJS

cl. D400, SW 7, length 500 mm

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

BG-FILCOTEN pro ductile-iron design-grate “VILLE“ NW 200, ductile-iron EN-GJS

cl. E600, SW 7, length 500 mm

with integrated longitudinal movement protection and fiX-lock, according to EN1433 - CE-certified

Anti-vandalism locking device made of stainless steel

cross piece for blocking the fiX-lock

BG-FILCOTEN pro ductile-iron slotted grating NW 200, ductile-iron EN-GJS

cl. D400, SW 14/200, length 500 mm

with integrated longitudinal movement protection, 4-fold screw-connection (only in cast-iron edge), according to EN1433 - CE-certified, bolting material must be ordered separately, bolting material is exchangeable.

BG-FILCOTEN pro ductile-iron bar grating NW 200, ductile-iron EN-GJS

cl. E600, MW 22/13, length 500 mm

with integrated longitudinal movement protection, 4-fold screw-conection (only in cast-iron edge), according to EN1433 - CE-certified, bolting material must be ordered separately, bolting material is exchangeable.

KIT-bolting material for ductile iron grating

steel, black galvanised - consisting of 1 pc. screw and 1 pc. nut (requirement: 8 pcs./m)

KIT-bolting material material for ductile iron grating

stainless steel A2 - consisting of 1 pc. screw and 1 pc. nut (requirement: 8 pcs./m)

- sump unit kit BG-FILCOTEN pro NW 200 made of FILCOTEN HPC (high-performance concrete)

with integrated edge made of galvanised steel, stainless steel 1.4301 or cast iron, incl. silt bucket

outlet: pipe coupling DN 200

Länge 500 mm / width 265 mm / height 750 mm

Delivery and installation according to the manufacturer's installation instructions.

Galvanised steel frame: 48,7 kg

Cast iron frame:	 49,1 kg

Stainless steel frame: 	 48,7 kg

- front cap closed, made of galvanised steel or stainless steel 1.4301

weight: 0,3 kg

- installation chair, Kit incl. screws for positioning the channel, galvanised steel

weight: 1,7 kg