- Heavy-duty drainage-channel BG-CLASSIC BGZ-S NW 300 made of concrete,

with integrated Z-shaped cast iron edge,

load-class A15 – F900 kN, according to EN1433 – CE-certified,

channel joint with safety seam, sealable,

length 1000 (2500) mm, without slope,

Fire-behaviour class: A1 according to EN13501 - non-combustible

100% recyclable with U-A marking,

Delivery and installation according to the manufacturer's installation instructions.

NW/No.:		LxWxH: [mm]		weight: [kg]

300 No. 0:		1000x399x395		169

300 CH 700:		2500x394x700		671

300 CH 700:		1000x394x700		269

- Heavy-duty drainage-channel as described in position 1, but

with bottom outlet DN 200 (Dm 226 mm),

NW/No.:		LxWxH: [mm]		weight: [kg]

300 No. 0:		1000x399x395		162

300 CH 700:		1000x394x700		258

- Covers/Grates made of galvanised steel or ductile iron

with integrated longitudinal movement protection and quick-locking system (SV),

load class C250kN – E600kN, according to EN1433 - CE-certified

BG-CLASSIC SV mesh grating NW 300, steel galvanised steel

cl. C250, MW 30/10, length 1000 mm,

with integrated longitudinal movement protection and quick-locking system (SV), according to EN1433 – CE- certified

BG-CLASSIC SV mesh grating NW 300, galvanised steel

cl. C250, MW 30/10, length 500 mm,

with integrated longitudinal movement protection and quick-locking system (SV), according to EN1433 – CE- certified

BG-CLASSIC SV ductile iron slotted grating NW 300, cast iron EN-GJS

cl. D400, SW 18/150, length 500 mm

with integrated longitudinal movement protection and quick-locking system (SV), according to EN1433 – CE- certified

BG-CLASSIC SV main bar grating NW 300, cast iron EN-GJS

cl. E600, MW 25/14, length 500 mm

with integrated longitudinal movement protection and quick-locking system (SV), according to EN1433 – CE- certified

- Covers/Grates made of cast iron inclusive bolting material 4-times,

with integrated longitudinal movement protection,

load class E600kN – F900kN, according to EN1433 – CE - certified

BG-CLASSIC main bar grating NW 300, cast iron EN-GJS

cl. E600, MW 25/14, length 500 mm

with integrated longitudinal movement protection, inclusive bolting material 4-times, according to EN1433 – CE- certified

BG-CLASSIC ductile iron slotted grating NW 300, cast iron EN-GJS

cl. F900, SW 16/148, length 500 mm

with integrated longitudinal movement protection, inclusive bolting material 4- times, according to EN1433 – CE- certified

Optional accessories cover:

SET- bolting material for mesh grating

steel, galvanised – consisting of 1 Stk. screw and 1 Stk. Mutter

SET- bolting material for cast grating

steel, black galvanised – consisting of 1 Stk. screw and 1 Stk. Mutter

- sump unit BG-CLASSIC BGZ-S NW 300 concrete, 2 parts

with integrated cast iron frame

outlet: RDS DN 200

length 550 mm / width 565 mm / height 990 mm

Delivery and installation according to the manufacturer's installation instructions.

weight:	214 kg

- silt bucket to sump unit, plastic

weight: 3,2 kg

- drain trap made of PVC duct bends DN 200/87°, plastic

weight: 7,4 kg

- foreheda-sealing for bottom outlet DN 200, Elastomer

weight: 0,6 kg

- front cap closed off galvanized steel

weight: 1,3 kg

- end cap with outlet DN 200 off galvanized steel

weight: 1,2 kg