- Safety channel BG-CLASSIC BG-SI NW 200 made of concrete,

with integrated special edge made of galvanized steel

load-class A15 – F900 kN, according to EN1433 – CE-certified,

channel joint with safety seam, sealable,

length 1000 (2500) mm, without slope,

Delivery and installation according to the manufacturer's installation instructions.

NW/No.:		LxWxH: [mm]		weight: [kg]

200 No. 0:		1000x612x485		560

200 No. 0:		2500x612x485		1400

- Covers/Grates made of cast iron inclusive bolting material 4-times,

with integrated longitudinal movement protection,

load class F900kN, according to EN1433 – CE - certified

BG-CLASSIC ductile iron slotted grating NW 200, cast iron EN-GJS

cl. F900, SW 16/220, length 500 mm

with integrated longitudinal movement protection, inclusive bolting material 4- times, according to EN1433 – CE- certified

BG-CLASSIC ductile iron cover NW 200, cast iron EN-GJS

cl. F900, length 500 mm

with integrated longitudinal movement protection, inclusive bolting material 4- times, according to EN1433 – CE- certified

Optional accessories cover:

SET- bolting material for cast grating

steel, black galvanised – consisting of 1 Stk. screw and 1 Stk. Mutter

- sump unit BG-CLASSIC BG-SI NW 200 concrete with integrated cast iron frame

procedure: RDS DN 200

length 1000 mm / width 600 mm / height 1000 mm

Delivery and installation according to the manufacturer's installation instructions.

weight:	1300 kg

- front cap closed out of galvanized steel

weight: 2,1 kg

- end cap with outlet DN 200 out of galvanized steel

weight: 1,5 kg